2.3 Synthesis of Previous Studies and Literature Matrix

The comparative analysis of five primary security paradigms identified in literature:

| Security Paradigm | Method | Primary Strengths | Major Limitations |
| --- | --- | --- | --- |
| Security Paradigm | Core Technology / Method | Primary Strengths | Major Limitations |
| Lightweight Crypto | ECC, ZKP, HMAC, Bitwise XOR | Reduced key size, strong mathematical security | Non-negligible bit-overhead and transmission payload size |
| PUF & Hardware Security | SRAM power-up states, Arbiter PUFs, BCH Fuzzy Extractors | High resistance to physical key extraction and cloning | Response sensitivity to temperature and voltage noise |
| Blockchain & Distributed Ledgers | Smart contracts, Merkle trees, IOTA Tangle | Eliminates single points of failure, transparent audit logs | High transaction latency, storage accumulation over time |
| Physical Layer Security | RF Fingerprinting (RFFI), CSI, Intelligent Reflecting Surfaces | Bypasses cryptographic key management entirely | Sensitive to physical user mobility, fast fading, and signal loss |
| Machine Learning & AI | Deep Neural Networks (DNN), Random Forest, Federated Learning | Context-aware, high accuracy (>97%), continuous scoring | High training/computational cost, vulnerable to noisy data |

The summary table detailing research objectives, testbed, method, evaluation metrics, key findings and identified limitations:

| Author(s) & Year | Core Focus / Objective | Dataset / Testbed | Method / Security Process | Evaluation Metrics | Key Findings | Research Limitations & Gaps |
| --- | --- | --- | --- | --- | --- | --- |
| Samarendra Nath Sur (2022) | Decentralized authentication via embedded blockchain | Public network / Localized servers | Embedded blockchain algorithms with localized offloading servers | Reliability, access, computational load reduction | Heavy computational data offloaded; secure decentralized ledger transactions | Lacks concrete performance metrics, latency values, and empirical data |
| Amar N. Alsheavi (2025) | Systematic review of IoT Authentication Protocols (APs) | Literature corpus review | Systematic review & comparative performance analysis | Privacy, scalability, human factors, efficacy | Comprehensive comparative analysis of existing APs and deployment roadmap | Pure review paper; presents no new benchmark data or proposed protocols |
| Junqing Zhang (2023) | Physical-layer identification before packet decoding | Simulated RF signals | RFFI using engineered features and hybrid protocol | Physical-layer classification accuracy | Leverages unique hardware features without heavy crypto computation | RFF signatures affected by environmental noise and component aging |
| Alhusenat, Ahmad Y.; Owida, Hamza Abu; Rababah, Hana A.; Al-Nabulsi, Jamal I.; Abuowaida, Suhaila (2023) | Real-time security for sensing data acquisition | Randomly distributed sensor data | One-Time Pad (OTP) with dynamic key generation across tiers | Efficacy against desynchronization, key creation speed | Dynamic encryption and lightweight auth resilient to desynchronization | Relies heavily on sensor randomness and specific hardware assumptions |
| Amar N. Alsheavi (2024) | Analysis of trends and gaps in IoT APs | Literature survey | Survey focusing on blockchain, ML, and interoperabilityInteroperability, platform limits, human factors | Interoperability, platform limits, human factors | Identified blockchain and ML as key solutions for future IoT APs | Lacks new experimental benchmark results or concrete protocols |
| Puja Sarkar & Amitava Nag (2024) | Post-quantum secure decentralized mutual authentication | Smart home sensor & user data | Lattice-based cryptography integrated with Blockchain | Quantum resistance, mutual auth resilience | Secures inter/intra-domain communications against quantum threats | Higher computational complexity typical of lattice structures |
| Sirisha Uppuluri & G. Lakshmees wari (2022) | Smart home device-to-user key agreement | Smart home device interaction data | Modified Honey Encryption (MHE-IS-CPMT) with ECC | Energy, storage overhead, attack resistance | Secure 5-step key agreement with low energy and storage overhead | 5-step process introduces slight initial session establishment delay |
| Muddesar Iqbal (2022) | Zero-Trust continuous authentication to prevent spoofing | Network / Sensor simulation data | PUF-based Device Continuous Auth (PUFDCA) combining static & location data | Energy consumption, storage, spoofing resilience | Resists impersonation and DoS attacks with lightweight footprint | Continuous location tracking introduces privacy and network overhead |
| Geetha Manoharan (2024) | Lightweight IoT identification framework for sensor nodes | Simulated network sensor traffic | Blockchain-based IoT identification framework | Attack survival rate, identification success rate | Achieved 99% identification success rate across security checks | Lacks mathematical breakdown of key storage on low-power devices |
| Ali Shahidinej ad (2023) | Low-overhead quantum-resilient D2D authentication | Comparative performance testbed | Lattice-based crypto + Edge computing + Blockchain | Computation time, communication overhead, quantum resilience | Reduced device computation overhead by ~58% and communication by ~2% | Requires constantly available edge computing infrastructure |
| S. B. Sarvaiya and D. N. Satange (2022) | Node and device identity verification | Arduino-based IoT environment | IP binding, encryption/decryption for IoT node auth | Authentication performance, data protection | Framework successfully validates IP-based IoT nodes | Lacks detailed quantitative scalability metrics |
| B. Kim, S. Yoon and Y. Kang (2022) | Device authentication on open IoT platforms | OCF IoT open platform | PUF-based device authentication integrated into OCF platform | Security enhancement, platform authentication | Significantly improves open IoT platform security via PUFs | Evaluation restricted strictly to OCF platform environment |
| A. M. Gamundani , A. Phillips and H. N. Muyingi (2023) | Scalable auth architecture for smart home devices | Smart-home IoT application | Cloud-inspired architecture using DIKW model | Lightweight execution, computational efficiency | Efficient authentication architecture designed for smart homes | Lacks quantitative performance metrics in abstract |
| W. Yodthong and M. Munlin (2023) | Lightweight communication protocol for smart homes | Smart-home environment | Lightweight protocol using hash functions and master keys | Computation cost, message count, energy consumption | Protocol uses 3 symmetric encryptions and 8 hashes (9.71 µJ energy) | Evaluation focuses heavily on operations rather than large physical tests |
| V. K. Rai, S. Tripathy and J. Mathew (2023) | Lightweight PUF protocol for resource-constrained IoT | PUF-based IoT system simulation | LPA: Lightweight PUF-based authentication protocol | Mutual auth security, lightweight footprint | Demonstrates PUF viability for lightweight authentication | Lacks hardware testing across diverse hardware variations |
| C. Shang, J. Cao, T. Zhu, Y. Zhang, B. Niu and H. Li (2024) | Remote attestation for Class-1 IoT devices | Cluster of Arduino Mega boards | CADFA: Clock skew device fingerprinting + remote attestation | Tamarin verification, identification accuracy (%) | 100% identification accuracy; resists replay and MITM attacks | Evaluation restricted to a small cluster of Arduino Mega boards |
| W. Jing, L. Peng, H. Fu and A. Hu (2024) | Borderless IoT network authentication | IoT and cloud/wireless network | Zero Trust architecture combined with Radio Frequency Fingerprints | Borderless network auth effectiveness | Reduces reliance on explicit perimeter security boundaries | Requires further testing across varied wireless propagation states |
| R. Parmar and N. Gajjar (2025) | Context-aware security advisories for IoT | Application-specific IoT domains | TinyLLaMA-based expert system for security recommendations | Recommendation accuracy, context adaptation | Provides application-specific security recommendations | Operates as an advisory tool rather than an automated enforcement system |
| N. R. A. Princess and U. S. Premarathn e (2025) | Hardware-based authentication using logic gates | 4-bit to 16-bit Arbiter PUF simulation | Arbiter PUF design extended to 8-bit and 16-bit configurations | CRP generation, reliability, uniqueness, scalability | Supports up to 65,536 CRPs for 16-bit configuration with high reliability | Primarily simulation-based; requires physical IC validation |
| M. Khatua, P. S. Varma, N. Shaikh and S. Das (2025) | Serverless lightweight mutual authentication | Network simulation traffic | Mutual authentication using a one-way accumulator | Computation and communication overhead reduction | Reduces dependence on central servers during authentication | Requires evaluation across larger, highly dynamic networks |
| Deepak Kumar, Bhaskar Mondal (2024) | Device-to-device authentication in open environments | Simulation testbed | PUF-based D2D scheme using an auth server as intermediary | Resistance to info leakage and attack modeling | Prevents information leakage and physical spoofing attacks | Lacks comprehensive mathematical delay analysis |
| Rizka Reza Pahlevi, Vera Suryani, Hilal Hudan Nuha, Rahmat Yasirandi (2022) | Multi-factor authentication against brute-force | 3,276,800 brute-force attack logs | Two-factor auth combining RFID cards and fingerprint biometrics | Brute-force failure rate (0% success out of 3.2M attempts) | 100% brute-force attack rejection rate across physical logs | Requires multi-stage physical user interaction during login |
| Poornima M. Chanal, Mahabales hwar S. Kakkasageri (2023) | Context-aware device authentication | Contextual network traffic | BDI cognitive agent combined with Random Forest ML algorithm | Context-aware prediction accuracy | Belief set explicitly models context to drive authentication | Secure authentication is not fully achieved at edge/router levels |
| Wael Y. Rayes, Khouloud Samrouth, Nader Bakir (2024) | Unauthorized access reduction in smart homes | Smart home conceptual testbed | Blockchain paired with Vazka dynamic password generation | Access control effectiveness, data integrity | Dynamic passwords based on collected data enhance data integrity | Conceptual framework tested via simulation without hardware metrics |
| Munkenyi Mukhandi, Francisco Damião, Jorge Granjal, João P. Vilela (2022) | Lightweight D2D consensus authentication | Ganache/Truffle + CORE emulator (20-40 nodes) | Hashed identities in Merkle tree on permissioned blockchain | Latency (s), throughput (kb/s), CPU utilization (%) | Latency scaled near-linearly (~12–25s); faster than central MQTT | Evaluated on PC emulators; unaddressed on-chain data privacy |
| Weizheng Wang, Dusit Niyato, Zehui Xiong, Zhimeng Yin (2025) | Cross-technology device authentication | Cross-Technology Comm (CTC) WiFi signals | AUTHFi: CTC reconstruction of partial transmission waveforms | Accuracy (%), FPR, TPR | Achieved up to 94.2% maximum authentication accuracy | Accuracy drops under high device mobility and signal loss |
| Shanyao Ren, Yizhong Liu, Beiyuan Yu, Jianwei Liu, Dongyu Li (2024) | Identity authentication between devices and cloud | Simulated network traffic | Certificateless lightweight authentication protocol | Computation time (ms), communication cost (bits) | D2C execution: 2.46 ms / 1,248 bits; D2D execution: 2.364 ms / 2,176 bits | Vulnerable to full compromise under stolen physical key attacks |
| Ashish Umesh Khandait, Tanuja R. Pattanshetti (2025) | Passwordless server-device communication | Network packet dataset | Zero Knowledge Proof (ZKP) protocol combined with HMAC hashing | Accuracy, response time, memory, throughput | Authenticates devices without transmitting plain passwords | Lacks detailed experimental hardware profiling |
| Junjie Hu, Danfeng Sun, Wei Lu, Junwei Dong, Huifeng Wu (2025) | Distributed industrial device access | Industrial access request logs | Blockchain-enabled distributed auth using IOTA Tangle | Access response time (s), attack interception | Intercepts unauthorized industrial requests while meeting access latency | Lacks explicit performance comparisons with non-tangle ledgers |
| Shenyao Jiang, Hao Zhou, Wangqiu Zhou, Xinyu Wang, Zhenjiang Li, Yusheng Ji (2025) | Wireless power transfer system security | Magnetic-based WPT system testbed | FreAuth+: Magnetic frequency feature-based authentication | Accuracy (%), response time (~1.6s) | Achieved 96.23% accuracy across 60+ devices; resilient to interference | Requires magnetic coupling hardware capabilities |
| Naif Alzahrani (2025) | D2D security for constrained networks | ProVerif simulation environment | D2D protocol using ECC and SHA-256 hashing | Energy, processing time, message size | Outperforms existing methods in battery conservation and security | Tested purely via software simulation without real hardware |
| Wenzheng Li, Shengnan Zhang , Zhu Chen, Liu Sen (2023) | Cross-domain verification without central authority | Simulated network traffic | Decentralized protocol using Ethereum smart contracts | Latency, transaction cost, DoS resilience | Eliminates single points of failure via smart contracts | On-chain data growth over time introduces slight transaction delays |
| Brij B. Gupta, Akshat Gaurav, Kwok Tai Chui, Ching-Hsien Hsu (2022) | Defense against DoS attacks on low-power devices | Simulated network traffic | Identity-Based Encryption (IBE) and lightweight key distribution | DoS resistance, processing overload efficiency | Effectively mitigates DoS/DDoS attacks while protecting key data | Tested in software simulations rather than on physical microcontrollers |
| Deepak Kumar, Bhaskar Mondal (2022) | Resistance to physical tampering and cloning | PUF circuits, Xilinx, Synopsys simulation | PUF hardware circuit verification | PUF reliability, uniqueness, energy use | Strong resistance against cloning, spoofing, and physical tampering | Temperature/voltage variations introduce noise into PUF outputs |
| Sangjukta Das, Maheshwari Prasad Singh, Suyel Namasudra (2023 | Low-power healthcare IoT security | Healthcare IoT simulation | Lightweight hashing and bitwise XOR logic | Processing time, message file size, energy use | Operates significantly faster and consumes less power than standard crypto | Evaluated via software tools rather than deployment on medical devices |
| Pavlov Zouridakis, Sai Manoj Pudukotai Dinakarrao (2023) | Continuous biometric authentication under power loss | IMU walking sensor dataset | Intermittent execution aware gait-based authentication | EER, energy consumption, checkpointing overhead | Maintains auth state across repeated power interruptions | Gait patterns vary with user fatigue, requiring continuous retraining |
| Muhammad Nawaz Khan, Haseeb ur Rahman, Tariq Hussain, Bailin Yang, Saeed Mian Qaisar (2024) | Vehicular network spoofing protection | Simulated automotive traffic logs | IPFS and cryptographic hashes for connected vehicles | Latency (s), throughput, communication overhead | Improves authentication accuracy while lowering network overhead | Evaluated through simulated networks rather than automotive testbeds |
| Xinglu Li, Kaizhi Huang, Shaoyu Wang, Xiaoming Xu (2022) | Spoofing/Sybil defense in wireless nodes | IoT nodes at varied physical locations | CNN classification applied to Channel State Info (CSI) | Accuracy (%), SNR robustness | High classification accuracy; reduces node computational load | CSI signatures sensitive to physical environmental movement |
| Xiao Wei, Muhammad Naveed Aman, Biplab Sikdar (2023) | Lightweight generic-domain authentication | Simulated network traffic | Lightweight protocol validated via BAN logic and AVISPA | Delay (ms), payload size (bytes), attack coverage | Significantly reduces computational latency and packet payload size | Tested on network simulators rather than physical microcontrollers |
| Hao Yin, Yi Li, Zichao Guo, Bing Li (2022) | Identity authorisation for ultra-restricted nodes | Simulated IoT communication packets | Symmetric encryption, hash functions, nonces, PUFs | Computation time (ms), payload size, energy | Reduces computational delay and frame sizes while guaranteeing security | Tested on synthetic attack vectors rather than real deployments |
| Watanyou Yodthong, Mudarmeen Munlin (2023) | Offloading computation from constrained devices | Simulated data inputs (80 to 256 bits) | Smart Home Agent (SHA) acting as intermediary via DIKW | Computational overhead (0 operations on device) | Device requires 0 XOR/hash operations ( 0x+0y+0z0x+0y+0z cost) | Relies heavily on simulation; lacks real-world physical metrics |
| Attlee M. Gamudani, Amelia Phillips, Hippolyte N. Muyingi (2023) | Multi-factor user authentication over cloud | Design Science Research testbed | Rabin Cryptosystem with biometrics, smart cards, passwords | Computation time, communication overhead, throughput | Resists replay, MITM, and password-guessing attacks | Lacks validation on large-scale physical hardware deployments |
| Devishree Naidu, Niranjan K. Ray (2022) | Cross-domain authorisation via backscatter signals | AVISPA / HLPSL generated data | FedScatter: Backscatter communications + Federated Learning | Accuracy (97.66%), TPR (97.01%), FPR (0.73%) | High authentication accuracy without sharing local device identities | Performance depends on tag density; requires hardware calibration |
| Guoqiang Zhang, Qiwei Hu, Yu Zhang, Yueyue Dai, Tao Jiang (2024) | High-speed security for medical IoT | Wireless backscatter signal data | Lightweight ECC integrated with DNA cryptography | Key length (bits), processing time (ms) | 512-bit DNA encoding executed in 95 ms; outperforms RSA | Tested via software simulations rather than physical FPGA chips |
| Vijaya Kumar Vadladi, D Marshiana (2024) | Key-extraction defense without large CRP databases | Simulated data inputs (80 to 256 bits) | PEAK: SRAM/Arbiter PUF + BCH Fuzzy Extractor + AES-GCM | Total execution time (0.926 ms), cost (4,672 bits) | Achieved high SRAM (96%) and APUF (98.9%) intra-reliability | BCH error correction introduces hardware slice overhead on FPGAs |
| C. Gupta, G. Varshney, G. Kumar, & S. Ahlawat (2026) | Key-extraction defense without large CRP databases | ESP32-WROOM + Pynq-Z2 FPGA | Intelligent Reflecting Surfaces measuring RSS ratios | Accuracy (99.82–99.99%), latency (~100 ms) | Provides +4.6 dB signal boost; resists DoS and physical attacks | Fails under high device mobility (>60 km/h) |
| Xiao Wei, Muhammad Naveed Aman, and Biplab Sikdar (2026) | IRS-aided physical-layer authentication | 1,000 Monte Carlo simulation trials | Enhanced ECC 3-factor mutual authentication | Computation time (225.52 ms), overhead (2816 bits) | Corrects validation flaw; resists 13 attack vectors | Incurs slightly higher communication overhead than baseline |
| Ilyes Ahmim, Nacira Ghoualmi-Zine, Feriel Bouakkaz, Abderrezak Rachedi (2023) | Session recovery for vehicular data collection | Synthetic ITS traffic logs | Lightweight crypto (XOR/HMAC) + cubic spline polynomials | Computation cost (7 hash + 4 HMAC), size (4,736 bits) | Achieves lower computation cost; verified via Scyther tool | Higher communication bit cost required to preserve anonymity |
| Yaovi Ahadjitse (2025) | Anonymous D2D continuous authentication | Synthetic experimental test vectors | Modular ECC framework (ECDH, ECDSA, SHA-256) | Energy (3.2 mJ), latency (52 ms), memory (1.5 MB) | SECP256R curve achieved highest efficiency for low-power IoT | Evaluated in simulation; lacks post-quantum crypto integration |